Intelligent Dipstick (IDS)

Harold Sleeper

Table of Contents

Parts Needed

Setting Up Arduino IDE

Prepare Blues Wireless

Installing Arduino Libraries

Upload Arduino Programs

Parts Needed

- 1 ESP32 Dev Kit C v4

- 1 Screw Terminal Block Breakout Board for ES32 Dev Kit C

- 2 green LED lights

- 2 red LED lights

- 4 220-ohm resistors

- 1 MAX 6675 + K-Type Thermocouple

Refer to Capstone Gateway Diagram.pdf and Capstone Sensor Diagram.pdf as an assembly guide.

Setting Up Arduino IDE 1.8.19 for IDS Sensors and Gateway

Arduino’s IDE needs to be configured to compile its sketches appropriately for the ESP32.

** Disable any antivirus software temporarily for the next steps! **

- First in the Arduino IDE, go to the File menu and choose Preferences

This should open a Preferences window. Within the Preferences window near the bottom, should be a text field labeled Additional Boards Manager URLs.

Copy the next two lines into the text field and press OK.

https://dl.espressif.com/dl/package_esp32_index.json, https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

The two separate URLs should be separated by a comma as shown and as it appears in this text.

- Next in the Arduino IDE, go to the Tools menu, choose Board, then go to Boards Manager…

This should open the Boards Manager window.

Search for ESP32 and press install button for the ESP32 by Espressif Systems.

- Go to CP210x USB to UART Bridge VCP Drivers - Silicon Labs (silabs.com)

Near the top of the webpage should be a hyperlink labeled Downloads. Click it!

Choose CP210x Universal Windows Driver from the options.

Extract the downloaded zip file into a location of your choosing and within that folder open CP210xVCPInstaller_x64.exe (this assumes you are using a 64-bit computer, otherwise choose the …_x86 option)

Follow the on-screen installation instructions

Prepare Blues Wireless Notecard

- Please follow the instructions found here on the Blues Wireless Developer site for directions on installing the Blues Wireless Notecard onto the Blues Wireless Notecarrier-AF.

- Once the Notecarrier is assembled use the instructions found here to create a Notehub Project and obtain a ProductUID.

- Save the ProductUID for use later in this project.

Installing Arduino Libraries

- Go to the Arduino IDE Tools menu, then choose Manage Libraries

- Search for "Blues" in the input box and click the Install button next to the "Blues Wireless Notecard" result.

- Search for MAX6675 and install the library by Yurri Salimov

- Search for NimBLE and install the library by h2zero

Uploading Arduino Programs onto the ESP32 Dev Kit C v1 and Adafruit Feather

To install Capstone_Sensors.ino onto the ESP32 Dev Kit C:

- Connect the Dev Kit C to a USB port via micro-USB connector

- Open Capstone_Sensors.ino in Arduino IDE

- Navigate to the Tools menu in Arduino IDE, then choose Port and select the appropriate serial port associated with the newly connected Dev Kit C

- Within the Tools menu, choose Board menu, select ESP Arduino, and from that menu choose “ESP32 Dev Module”

- Navigate to the Sketch menu in Arduino IDE and select the Upload option (alternatively press Ctrl+U). The program will compile and upload onto the ESP32 board. *

To install Capstone_Gateway.ino onto the Adafruit Feather:

- Connect the Adafruit to a USB port via micro-USB connector (Pro Tip: disconnect other ESP32s from the same computer to avoid confusion as to which serial port to use)

- Open Capstone_Gateway.ino in Arduino IDE

- Go to line 8 in the code and use your own ProjectUID to replace the Product UID placeholder within the quotes. As seen below: 
#define productUID "com.your-company.your-name:you_project"

- Navigate to the Tools menu in Arduino IDE, then choose Port and select the appropriate serial port associated with the newly connected Adafruit

- Within the Tools menu, choose Board menu, select ESP Arduino, and from that menu choose “Adafruit ESP32 Feather”

- Navigate to the Sketch menu in Arduino IDE and select the Upload option (alternatively press Ctrl+U). The program will compile and upload onto the ESP32 board.

*Depending on the manufacturer, the Dev Kit C might require the user to press and hold the Boot button on the ESP32 board while Arduino IDE tries to connect to it. Release boot button when uploading proceeds.